REQUEST FOR QUOTE

Mobile devices for the Equipment Pre-Use Checklist System — pilot rollout

| Issuing organisation | Enaleni Engineering (Pty) Ltd · Belfast Coal Mine, Mpumalanga |
| --- | --- |
| RFQ reference | ENA-RFQ-EQ-2026-001 |
| Issued | _____ / _____ / 2026 |
| Quote due back by | _____ / _____ / 2026 (14 working days from issue) |
| Primary contact | TauGM · tau.maswanganye@gmail.com |
| Deployment site | Belfast Coal Mine, Mpumalanga, South Africa |

# 1.  Project context

Enaleni Engineering has built and deployed the Equipment Pre-Use Checklist System for Belfast Coal Mine: an ASP.NET Core web back-end with a .NET MAUI Blazor Hybrid mobile app targeting Android and Windows. The system enforces MHSA Section 22(a) operator competency at every pre-shift check, captures defects with photos and voice memos, and works fully offline with deferred sync. A mobile-device allowlist pins each device to a single user via a hardware fingerprint.

This RFQ covers the hardware fleet for a 50-machine pilot at Belfast: tablets for operators, supervisors, and mechanics; a small number of Windows 2-in-1s for admin and SHE staff; charging infrastructure; MDM licensing; and site enrolment.

# 2.  Hardware specification

Vendors must quote against the target specification in column 3. Example models in column 4 are illustrative only — equivalent products from any reputable manufacturer are acceptable. Substitutions must meet OR exceed every target spec.

| Item | Qty | Target specification (mandatory) | Example model (or equivalent) |
| --- | --- | --- | --- |
| Rugged 8-inch Android tablet (operators, in-cab use) | 20 | IP68 dust/water; MIL-STD-810H drop ≥1.5 m; sunlight-readable display ≥600 nit; Android 13+; ≥6 GB RAM, ≥128 GB storage; fingerprint sensor; replaceable battery; glove-capable touchscreen; rear ≥13 MP camera; microphone; Wi-Fi 6 + LTE; stylus included | Samsung Galaxy Tab Active 5 (or Tab Active 4 Pro) |
| Semi-rugged 11-inch Android tablet (supervisors, mechanics) | 6 | IP52 minimum (paired with case below); Android 13+; ≥6 GB RAM, ≥128 GB storage; 10.9–11" display; fingerprint sensor; ≥12 MP rear camera; Wi-Fi 6; stylus included | Samsung Galaxy Tab S9 FE (or Lenovo Tab P12) |
| Windows 2-in-1 tablet (admin, SHE officer, mine manager) | 2 | Windows 11 Pro; ≥16 GB RAM, ≥512 GB SSD; ≥13" display; Type Cover (keyboard) + stylus; rugged carry case rated to 1.2 m drop; biometric login (fingerprint or face); Wi-Fi 6E; 2 × USB-C with DisplayPort | Microsoft Surface Pro 11 (or Lenovo ThinkPad X12) |
| Rugged Android phone (spot inspections, mobile foremen) | 4 | IP68; MIL-STD-810H; Android 13+; ≥4 GB RAM; fingerprint sensor; programmable button; replaceable or extended battery; pocket-sized (≤6.6" display); ≥13 MP camera | Samsung Galaxy XCover7 (or Cat S75) |

# 3.  Accessories and ancillary

- Spare batteries — minimum 4 per Tier-1 tablet so shift-end hot-swap is possible at all assigned machines.

- Screen protectors — tempered glass on every device, fitted before delivery.

- Cases — Tier-2 tablets shipped with OtterBox Defender or equivalent rated to MIL-STD-810H.

- Charging cart — single locked 20-bay cart for the Tier-1 tablets at shift-change point. Per-bay status LEDs.

- Asset tags — each device pre-labelled with a printed asset tag bonded to the case. Tag format to be agreed.

# 4.  Mobile device management

The application uses a hardware-fingerprint allowlist that pins each device to a single operator. Centralised remote-wipe, app-deployment, and configuration push are mandatory. Quote on 12 months of MDM licensing per device.

- Android tier: Samsung Knox Mobile Enrolment or Microsoft Intune (vendor to specify).

- Windows tier: Microsoft Intune via existing Microsoft 365 tenant.

- Initial enrolment + zero-touch provisioning must be included in the line item.

# 5.  Service, warranty, and support

- Warranty — minimum 24 months on tablets and phones, 12 months on accessories, on-site replacement within 5 working days.

- Hot-spare pool — vendor to hold a 10 % hot-spare pool at their location during the first 6 months.

- Delivery — to Belfast Coal Mine site office, scheduled with the operations manager.

- Handover training — two days on site for IT and SHE staff — enrolment workflow, allowlist management, fault-reporting process.

- Lead time — from PO to delivery on site must be stated explicitly per line item; affects scoring.

# 6.  Vendor pricing response

Complete every line of the table below. Sub-total each row and total at the foot. Where a substitution against the example model is being proposed, attach a one-page comparison sheet showing how the substitution meets or exceeds the target spec.

| Line item | Qty | Unit price (ZAR) | Lead time | Subtotal (ZAR) |
| --- | --- | --- | --- | --- |
| Rugged 8" Android tablet — Tier 1 (operators) |  |  |  |  |
| Semi-rugged 11" Android tablet — Tier 2 (supervisors / mechanics) |  |  |  |  |
| Windows 2-in-1 tablet — Tier 3 (admin / SHE / mine manager) |  |  |  |  |
| Rugged Android phone — spot inspections |  |  |  |  |
| Spare batteries (Tier 1) |  |  |  |  |
| Rugged case + screen protector — Tier 2 |  |  |  |  |
| 20-bay charging cart |  |  |  |  |
| MDM licence — 1 year per device |  |  |  |  |
| Asset tagging + initial enrolment (one-off) |  |  |  |  |
| Delivery to site (Belfast, Mpumalanga) |  |  |  |  |
| On-site setup + handover training (2 days) |  |  |  |  |
| TOTAL (ex VAT) |  |  |  |  |

# 7.  Evaluation criteria

Quotes will be scored against the following weighted criteria. The lowest-priced quote will not automatically win; total cost of ownership, lead time, and support responsiveness all materially affect the score.

- Compliance with target spec (40 %) — every mandatory item met, evidence attached for any substitution.

- Total landed cost over 3 years (30 %) — hardware + MDM + warranty + projected replacement.

- Lead time and stock availability (15 %) — shorter is better; backorders disqualify.

- Service responsiveness and on-site presence in Mpumalanga (10 %) — documented SLA.

- Local-content and B-BBEE contribution (5 %) — stated rating with supporting affidavit.

# 8.  Submission

Submit responses by email to the primary contact named on the cover. Subject line must read "ENA-RFQ-EQ-2026-001 — [Vendor name]". Quotes must include the completed pricing table, a covering letter on letterhead, B-BBEE certificate, and any substitution evidence sheets. Late submissions will not be considered.

Enaleni Engineering reserves the right to award the contract in whole or in part to one or multiple vendors, to negotiate further with shortlisted respondents, and to decline all quotes without giving reasons.

Issued on behalf of Enaleni Engineering (Pty) Ltd

TauGM — Technical Lead, Equipment Checklist Programme

tau.maswanganye@gmail.com